实验数据汇总表

基于 ZYNQ-7010 的 INT8 量化神经网络无线信号调制识别加速系统

用途：可直接用于项目报告、README、答辩材料和简历数据核对。建议后续所有材料统一引用本文件中的最终数字。

1. 模型精度与量化结果

| 模型/指标 | 结果 |
| --- | --- |
| FP32 Test Accuracy | 71.77% |
| INT8 Test Accuracy | 70.83% |
| FP32 → INT8 Accuracy Loss | 0.94 pct |
| FP32 / INT8 Class Agreement | 91.08% |
| INT8 Accuracy @ SNR ≥ 0 dB | 75.03% |
| INT8 Accuracy @ SNR ≥ 6 dB | 79.63% |
| INT8 Accuracy @ SNR ≥ 10 dB | 81.47% |

说明：INT8 PTQ 后测试集准确率由 71.77% 降至 70.83%，精度损失仅 0.94 个百分点。

2. HLS Baseline 与最终优化版本综合结果

| 指标 | Baseline V0 | Optimized | 变化 |
| --- | --- | --- | --- |
| Target Clock | 10.000 ns | 10.000 ns | — |
| Estimated Clock | 8.541 ns | 9.634 ns | 均满足 100 MHz |
| Total Latency | 229870 cycles | 154100 cycles | -32.96% |
| HLS 计算延迟 @100 MHz | 2.2987 ms | 1.5410 ms | -32.96% |
| 理论吞吐 @100 MHz | 435.03 fps | 648.93 fps | +49.17% |
| BRAM18K | 9 | 16 | +7 |
| DSP48E | 7 | 19 | +12 |
| FF | 1260 | 2807 | +1547 |
| LUT | 2284 | 4745 | +2461 |

2.1 最终优化版本资源占用（XC7Z010）

| Resource | Used | Available | Utilization |
| --- | --- | --- | --- |
| BRAM18K | 16 | 120 | 13.3% |
| DSP48E | 19 | 80 | 23.8% |
| FF | 2807 | 35200 | 8.0% |
| LUT | 4745 | 17600 | 27.0% |

说明：优化通过增加并行计算资源换取延迟下降，最终资源占用仍处于 XC7Z010 可接受范围内。

3. Conv2 核心优化过程

| 指标 | Baseline | Parallel5 V1 | Optimized Pair |
| --- | --- | --- | --- |
| Conv2 Latency | 180224 cycles | 118784 cycles | 104448 cycles |
| Total Latency | 229870 cycles | 168436 cycles | 154100 cycles |
| Estimated Clock | 8.541 ns | 9.634 ns | 9.634 ns |
| DSP | 7 | 14 | 19 |
| BRAM | 9 | 16 | 16 |
| LUT | 2284 | 3716 | 4745 |

说明：最终 Conv2 latency 由 180224 cycles 降至 104448 cycles，下降约 42.0%。

4. FPGA 实板性能测试

测试条件：100 MHz；10 帧 warm-up；1000 帧计时；AXI DMA Simple Mode；输入 256 B/帧；输出 24 B/帧。

| FPGA版本 | 平均延迟 | 最小延迟 | 最大延迟 | Throughput |
| --- | --- | --- | --- | --- |
| Baseline FPGA | 2302.832 us | 2302.775 us | 2303.006 us | 434.24 fps |
| Optimized FPGA | 1545.035 us | 1545.017 us | 1545.098 us | 647.23 fps |

| 指标 | 改善 |
| --- | --- |
| Average Latency Reduction | 32.91% |
| Throughput Improvement | 49.05% |
| Throughput Ratio | 1.49× |

5. HLS 估算与实板实测对比

| Version | HLS估算延迟 | 实板端到端延迟 | 额外开销 |
| --- | --- | --- | --- |
| Baseline | 2298.700 us | 2302.832 us | ≈ 4.132 us |
| Optimized | 1541.000 us | 1545.035 us | ≈ 4.035 us |

说明：实板测得延迟与 HLS cycle-based estimation 高度一致，DMA、Cache maintenance 与 PS 控制的额外单帧开销约 4 μs。

6. Cortex-A9 与 FPGA 性能对比

ARM 测试条件：Cortex-A9；Standalone/bare-metal；GCC -O2；10 帧 warm-up；1000 帧计时；INT8/INT32 整数计算。

| Platform | Average Latency | Throughput | Speedup vs ARM |
| --- | --- | --- | --- |
| Cortex-A9 INT8 C | 13.524302 ms | 73.94 fps | 1.00× |
| FPGA Baseline | 2.302832 ms | 434.24 fps | 5.87× |
| FPGA Optimized | 1.545035 ms | 647.23 fps | 8.75× |

| 对比指标 | 结果 |
| --- | --- |
| Optimized FPGA vs ARM latency reduction | 88.58% |
| Optimized FPGA speedup vs ARM | 8.75× |
| Baseline FPGA speedup vs ARM | 5.87× |

7. Bit-Exact 正确性验证

| 验证阶段 | 结果 |
| --- | --- |
| Python Integer Reference | PASS |
| HLS C Simulation | PASS |
| HLS C/RTL Co-Simulation | PASS |
| FPGA Fixed Vector | 6 / 6 logits exact |
| Cortex-A9 Fixed Vector | 6 / 6 logits exact |
| FPGA 100-vector Regression | 600 / 600 logits exact |
| FPGA Bit-exact Frames | 100 / 100 |
| FPGA Class Agreement | 100 / 100 |
| Cortex-A9 100-vector Regression | 600 / 600 logits exact |
| Cortex-A9 Bit-exact Frames | 100 / 100 |

Python INT8 Reference = Cortex-A9 INT8 C = HLS C = RTL Co-Sim = ZYNQ-7010 FPGA

8. 项目核心数据总表

| Category | Baseline / Reference | Final Optimized Result |
| --- | --- | --- |
| FP32 Accuracy | 71.77% | — |
| INT8 Accuracy | — | 70.83% |
| Quantization Loss | — | 0.94 pct |
| HLS Latency | 229870 cycles | 154100 cycles |
| HLS Latency Reduction | — | 32.96% |
| Estimated Clock | 8.541 ns | 9.634 ns |
| DSP | 7 | 19 / 80 |
| BRAM | 9 | 16 / 120 |
| LUT | 2284 | 4745 / 17600 |
| FPGA Measured Latency | 2.302832 ms | 1.545035 ms |
| FPGA Throughput | 434.24 fps | 647.23 fps |
| FPGA Throughput Improvement | — | 49.05% |
| Cortex-A9 Latency | 13.524302 ms | — |
| Optimized FPGA vs ARM | — | 8.75× speedup |
| Multi-vector Hardware Regression | — | 600 / 600 logits exact |
| Bit-exact Frames | — | 100 / 100 PASS |

9. 建议对外统一引用的核心结论

INT8 测试准确率 70.83%，相较 FP32 仅下降 0.94 个百分点。

HLS latency 由 229870 cycles 降至 154100 cycles，降低 32.96%。

实板平均延迟由 2302.832 us 降至 1545.035 us，降低 32.91%。

实板吞吐由 434.24 fps 提升至 647.23 fps，提升 49.05%。

优化 FPGA 相对 Cortex-A9 bare-metal INT8 C 实现达到 8.75× 加速。

100 个不同样本实现 600/600 logits exact、100/100 bit-exact frames。